COMPANY

GENERIC RISK ASSESSMENT (GRA) – TRACTOR MOUNTED FLAIL

Activity / Task: Operation, maintenance and transport of tractor mounted flail / mulcher units

Document Ref: ARB-GRA-07

Version: ____    Date Issued: ____    Review Date: ____

Assessed by: ______________________________

## 1. Scope of Risk Assessment

This Generic Risk Assessment (GRA) covers the operation, maintenance, refuelling, transport and environmental considerations associated with tractor mounted flail or mulcher units. This includes roadside vegetation management, site clearance and working on uneven or sloping terrain. This GRA must be read alongside a Site-Specific Risk Assessment (SSRA).

## 2. Responsibilities

Management is responsible for ensuring operators are trained and competent, equipment is maintained in accordance with manufacturer guidance, and safe systems of work are implemented.

Operators are responsible for conducting daily pre-start inspections, operating the machinery safely, establishing exclusion zones and reporting defects immediately.

## 3. Persons at Risk

☒ Employees    ☒ Contractors    ☒ Members of the Public    ☒ Other Road users    ☒ Environment

## 4. PPE Requirements

Mandatory PPE: High visibility clothing (EN ISO 20471), safety footwear, hearing protection (EN 352), eye protection (EN 166), gloves and suitable task-specific PPE as identified in the SSRA.

## 5. Emergency Arrangements

A site-specific emergency plan must be completed prior to works commencing in accordance with FISA 802. At least one operative on site must hold Emergency First Aid at Work (+F Forestry where applicable). A fire extinguisher must be mounted in the tractor cab. Spill kits must be available when working near watercourses or environmentally sensitive areas.

## 6. Risk Matrix & Scoring

Risk is assessed using Likelihood (L) and Severity (S). Each rating must show the component scores and the calculation: L × S = Risk Rating. Initial risk is assessed before controls. Residual risk is reassessed after the listed controls using a new Residual L and Residual S score; these must not be copied from the initial assessment without review.

| Severity ↓ / Likelihood → | 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Very Likely |
| --- | --- | --- | --- | --- | --- |
| 1 Minor | 1 | 2 | 3 | 4 | 5 |
| 2 First Aid | 2 | 4 | 6 | 8 | 10 |
| 3 Medical | 3 | 6 | 9 | 12 | 15 |
| 4 Major | 4 | 8 | 12 | 16 | 20 |
| 5 Fatal | 5 | 10 | 15 | 20 | 25 |

Risk bands: 1–4 Low | 5–9 Medium | 10–16 High | 17–25 Intolerable

## 7. Relevant UK Legislation, Guidance & Good Practice

Health and Safety at Work etc. Act 1974

Management of Health and Safety at Work Regulations 1999

Provision and Use of Work Equipment Regulations 1998 (PUWER)

Control of Noise at Work Regulations 2005

Control of Vibration at Work Regulations 2005

Electricity at Work Regulations 1989

Road Traffic Act 1988 (where working on highways)

Environmental Protection Act 1990

FISA 204 – Flails and Mulchers in Tree Work

FISA 802 – Emergency Planning

FISA 804 – Electricity at Work Forestry & Arboriculture

FISA 805 – Training and Certification

HSE INDG229 – Using Work Equipment Safely

## 8. Hazards & Control Measures

| Hazard | Persons at Risk | How Harm May Occur | Initial L | Initial S | Initial L×S | Control Measures | Res. L | Res. S | Res. L×S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Entanglement in moving parts | Operator | Severe injury from contact with rotating flail head or PTO shaft | 3 | 5 | 15 | Guards fitted and maintained; PTO guarded; machine shut down and isolated before maintenance; trained operators only; lock-off procedure followed. | 1 | 5 | 5 |
| Ejection of debris | Operator, public | Stones, metal or glass projected causing injury | 3 | 5 | 15 | Exclusion zone established; banksman positioned safely; work stopped if public enter zone; head not slewed into carriageway; appropriate shielding fitted. | 1 | 5 | 5 |
| Tractor overturn on slope | Operator | Crush injury or fatality | 3 | 5 | 15 | ROPS fitted; seatbelt worn at all times; slopes assessed before operation; traverse slopes where possible; avoid excessive gradient; competent operator only. | 1 | 5 | 5 |
| Runaway vehicle | Operator, others | Collision or crush injury | 3 | 5 | 15 | Park on level ground; handbrake applied; keys removed when unattended; wheels chocked where necessary. | 1 | 5 | 5 |
| Hydraulic hose failure | Operator, environment | High-pressure injection injury or environmental contamination | 3 | 5 | 15 | Hydraulic lines inspected daily; defective hoses replaced immediately; spill kits available; exclusion zone maintained; environmental assessment completed. | 1 | 5 | 5 |
| Noise exposure | Operator | Hearing damage | 3 | 3 | 9 | Hearing protection provided and worn; equipment maintained; exposure assessed under Noise Regulations. | 1 | 3 | 3 |
| Whole body vibration | Operator | Musculoskeletal disorders | 3 | 3 | 9 | Equipment maintained; suitable seating and suspension used; exposure duration managed; health surveillance where required. | 1 | 3 | 3 |
| Working adjacent to highway | Operator, public | Collision with passing traffic | 3 | 5 | 15 | Traffic management implemented where required; flashing beacons used; high visibility clothing worn; head not extended into carriageway; compliance with Chapter 8 where applicable. | 1 | 5 | 5 |
| Fire during operation or refuelling | Operator, environment | Burns or machinery loss | 3 | 4 | 12 | No smoking policy; engine cooled before refuelling; approved fuel containers; fire extinguisher in cab; debris cleared from hot surfaces. | 1 | 4 | 4 |
| Struck by moving tractor during manoeuvre | Employees, public | Impact injury | 3 | 5 | 15 | 360° awareness maintained; banksman used where visibility restricted; reversing alarms fitted; clear communication agreed before manoeuvre. | 1 | 5 | 5 |

## 9. Review & Monitoring

This GRA must be reviewed annually, following any incident, introduction of new equipment, changes in legislation or following audit recommendations.