COMPANY

GENERIC RISK ASSESSMENT (GRA) – HEDGE WORK OPERATIONS

Activity / Task: Hedge trimming, hedge reduction and hedge removal operations using hand tools and powered equipment

Document Ref: ARB-GRA-09

Version: ____    Date Issued: ____    Review Date: ____

Assessed by: ______________________________

## 1. Scope of Risk Assessment

This Generic Risk Assessment (GRA) covers hedge trimming, reduction and removal works including use of hand tools (loppers, secateurs, saws), petrol or battery hedge trimmers, pole hedge trimmers, step ladders and associated manual handling and waste removal. Hedge removal involving chainsaw use must also be read in conjunction with the relevant Chainsaw GRA. Work from MEWPs must be read alongside the MEWP GRA. This document must be used with a Site-Specific Risk Assessment (SSRA).

## 2. Responsibilities

Management must ensure operators are trained and competent, equipment is maintained in accordance with PUWER and manufacturer guidance, and adequate supervision is provided.

The Team Leader is responsible for completing and briefing the SSRA, establishing exclusion zones and monitoring site conditions. Operators are responsible for safe operation, correct PPE use and reporting hazards immediately.

## 3. Persons at Risk

☒ Employees    ☒ Contractors    ☒ Members of the Public    ☒ Clients    ☒ Other road users

## 4. PPE Requirements

Mandatory PPE: Safety helmet where overhead risk exists (EN 397), eye protection (EN 166), hearing protection (EN 352 where required), suitable gloves, safety footwear with good grip, and high visibility clothing when working near highways.

## 5. Emergency Arrangements

Emergency arrangements must comply with FISA 802. At least one operative must hold Emergency First Aid at Work (+F Forestry where chainsaw use is involved). A first aid kit must be available on site. Nearest A&E and precise site location (including What3Words where appropriate) must be recorded.

## 6. Risk Matrix & Scoring

Risk is assessed using Likelihood (L) and Severity (S). Each rating must show the component scores and the calculation: L × S = Risk Rating. Initial risk is assessed before controls. Residual risk is reassessed after the listed controls using a new Residual L and Residual S score; these must not be copied from the initial assessment without review.

| Severity ↓ / Likelihood → | 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Very Likely |
| --- | --- | --- | --- | --- | --- |
| 1 Minor | 1 | 2 | 3 | 4 | 5 |
| 2 First Aid | 2 | 4 | 6 | 8 | 10 |
| 3 Medical | 3 | 6 | 9 | 12 | 15 |
| 4 Major | 4 | 8 | 12 | 16 | 20 |
| 5 Fatal | 5 | 10 | 15 | 20 | 25 |

Risk bands: 1–4 Low | 5–9 Medium | 10–16 High | 17–25 Intolerable

## 7. Relevant UK Legislation, Guidance & Good Practice

Health and Safety at Work etc. Act 1974

Management of Health and Safety at Work Regulations 1999

Provision and Use of Work Equipment Regulations 1998 (PUWER)

Work at Height Regulations 2005

Control of Noise at Work Regulations 2005

Control of Vibration at Work Regulations 2005

Electricity at Work Regulations 1989

HSE INDG229 – Using Work Equipment Safely

HSE INDG402 – Safe Use of Ladders and Step Ladders

FISA 802 – Emergency Planning

FISA 804 – Electricity at Work Forestry & Arboriculture

FISA 805 – Training and Certification

## 8. Hazards & Control Measures

| Hazard | Persons at Risk | How Harm May Occur | Initial L | Initial S | Initial L×S | Control Measures | Res. L | Res. S | Res. L×S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cuts from blades (manual or powered) | Operator | Lacerations from hedge trimmer or hand tools | 3 | 4 | 12 | Operators trained and competent; two-handed operation; guards in place; PPE worn; equipment maintained; isolation before maintenance. | 1 | 4 | 4 |
| Ejection of debris | Operator, public | Eye injury from thrown material | 3 | 4 | 12 | Exclusion zone established (minimum 5m); eye protection worn; public kept clear; work stopped if zone breached. | 1 | 4 | 4 |
| Working at height (steps/ladders) | Operator, assistant | Fall from ladder causing fracture or head injury | 3 | 4 | 12 | Use of ladders minimised; ladder inspected pre-use; stable ground selected; assistant footing ladder; no overreaching; compliance with WAHR. | 1 | 4 | 4 |
| Manual handling of hedge arisings | Operator | Back strain or musculoskeletal injury | 3 | 3 | 9 | Work planned to reduce lifting; mechanical aids used where possible; correct lifting techniques applied; task rotation implemented. | 1 | 3 | 3 |
| Use of pole hedge trimmer | Operator | Muscle strain or falling material striking operator | 3 | 4 | 12 | Small sections removed; operator positioned outside drop zone; correct head angle selected; two-handed operation maintained. | 1 | 4 | 4 |
| Contact with overhead utilities | Operator | Electric shock or fatality | 2 | 5 | 10 | Overhead cables identified during SSRA; minimum approach distances maintained; FISA 804 guidance followed; isolation confirmed where required. | 1 | 5 | 5 |
| Noise exposure | Operator | Hearing damage | 3 | 3 | 9 | Hearing protection provided and worn; exposure assessed; equipment maintained. | 1 | 3 | 3 |
| Hand-arm vibration | Operator | HAVS or VWF | 3 | 3 | 9 | Low vibration equipment selected; exposure monitored; health surveillance implemented where required. | 1 | 3 | 3 |
| Fuel handling and fire risk | Operator, public | Burns or fire during refuelling | 3 | 4 | 12 | Refuel in well-ventilated area; engine cooled before refuelling; approved containers used; no smoking; fire extinguisher available. | 1 | 4 | 4 |
| Biological hazards (sharps, animal waste) | Operator | Infection from contaminated material | 3 | 3 | 9 | Visual inspection before cutting; tools used to expose ground; sharps reported, and specialist removal arranged; hands washed before eating; cuts cleaned and dressed. | 1 | 3 | 3 |

## 9. Review & Monitoring

This GRA must be reviewed annually, following any incident, introduction of new equipment, changes in legislation or audit recommendations.