COMPANY

GENERIC RISK ASSESSMENT (GRA) – NOISE EXPOSURE

Activity / Task: Exposure to noise from plant, tools and machinery during arboricultural and grounds maintenance operations

Document Ref: ARB-GRA-15

Version: ____    Date Issued: ____    Review Date: ____

Assessed by: ______________________________

## 1. Scope of Risk Assessment

This Generic Risk Assessment (GRA) covers foreseeable hazards arising from occupational exposure to noise generated by powered plant, tools and machinery. It applies to all employees and others who may be exposed to harmful noise levels during arboricultural and grounds maintenance activities. This GRA must be read alongside a Site-Specific Risk Assessment (SSRA).

## 2. Responsibilities

Management must comply with the Control of Noise at Work Regulations 2005, ensure suitable and sufficient noise risk assessments are undertaken, implement control measures in line with the hierarchy of control, and provide appropriate hearing protection and health surveillance where required.

Team Leaders must monitor compliance with hearing protection requirements and rotate staff where necessary to limit exposure duration.

Employees must wear hearing protection where required, report defects in PPE, and attend health surveillance appointments.

Noise exposure assessments and calculations shall be recorded and retained where exposure is likely to meet or exceed the Upper Exposure Action Value.

## 3. Persons at Risk

☒ Employees    ☒ Apprentices    ☒ Contractors    ☐ Members of the Public

## 4. PPE Requirements

Hearing protection must meet EN 352 standards and be selected based on suitable SNR rating to reduce exposure below legal limits. PPE must not over-attenuate where communication is required for safety.

## 5. Emergency Arrangements

Where excessive or unusual noise is detected indicating equipment malfunction, work must stop immediately, and the equipment isolated. First aid arrangements must comply with FISA 802. Any suspected acute hearing damage must be reported and referred for medical assessment.

## 6. Risk Matrix & Scoring

Risk is assessed using Likelihood (L) and Severity (S). Each rating must show the component scores and the calculation: L × S = Risk Rating. Initial risk is assessed before controls. Residual risk is reassessed after the listed controls using a new Residual L and Residual S score; these must not be copied from the initial assessment without review.

| Severity ↓ / Likelihood → | 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Very Likely |
| --- | --- | --- | --- | --- | --- |
| 1 Minor | 1 | 2 | 3 | 4 | 5 |
| 2 First Aid | 2 | 4 | 6 | 8 | 10 |
| 3 Medical | 3 | 6 | 9 | 12 | 15 |
| 4 Major | 4 | 8 | 12 | 16 | 20 |
| 5 Fatal | 5 | 10 | 15 | 20 | 25 |

Risk bands: 1–4 Low | 5–9 Medium | 10–16 High | 17–25 Intolerable

## 7. Relevant UK Legislation, Guidance & Good Practice

Health and Safety at Work etc. Act 1974

Management of Health and Safety at Work Regulations 1999

Control of Noise at Work Regulations 2005

Provision and Use of Work Equipment Regulations 1998 (PUWER)

HSE L108 – Controlling Noise at Work

HSE INDG362 – Noise at Work

FISA 802 – Emergency Planning

FISA 805 – Training and Certification

## 8. Exposure Action and Limit Values

Lower Exposure Action Value (LEAV): 80 dB(A) – Information and training required.

Upper Exposure Action Value (UEAV): 85 dB(A) – Noise reduction programme and mandatory hearing protection.

Exposure Limit Value (ELV): 87 dB(A) – Must not be exceeded (taking account of hearing protection).

## 9. Typical Equipment Noise Levels (Indicative)

| Equipment | Typical Noise Level (dB) | Risk Category |
| --- | --- | --- |
| Chainsaw (Petrol) | 100–115 dB | High |
| Brush Cutter / Strimmer | 95–105 dB | High |
| Wood Chipper | 100–110 dB | High |
| Hedge Trimmer | 95–105 dB | High |
| Leaf Blower | 90–102 dB | Medium-High |
| Stump Grinder | 100–112 dB | High |

## 10. Hazards & Control Measures

| Hazard | Persons at Risk | How Harm May Occur | Initial L | Initial S | Initial L×S | Control Measures | Res. L | Res. S | Res. L×S |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Exposure above UEAV (≥85 dB) | Operators | Long-term hearing damage (NIHL) | 4 | 4 | 16 | Substitution for quieter equipment where possible; task rotation; hearing protection mandatory; signage displayed; supervision enforced. | 1 | 4 | 4 |
| Exposure above ELV (87 dB) | Operators | Immediate and irreversible hearing damage | 3 | 5 | 15 | Exposure calculations undertaken; SNR selected correctly; exposure duration controlled; equipment maintained; monitoring records retained. | 1 | 5 | 5 |
| Failure to wear hearing protection | Operators | Cumulative hearing damage | 3 | 4 | 12 | Mandatory PPE policy; training in correct fitting; supervision and enforcement; defective PPE replaced immediately. | 1 | 4 | 4 |
| Poor equipment maintenance increasing noise | Operators | Excess noise due to worn components | 3 | 3 | 9 | Maintenance regime followed; defective silencers replaced; documented inspections retained under PUWER. | 1 | 3 | 3 |
| Lack of awareness of noise risk | Operators | Unsafe exposure due to misunderstanding of risk | 3 | 3 | 9 | Noise awareness training included in induction; annual refresher toolbox talks; clear signage on high-noise equipment. | 1 | 3 | 3 |
| Failure to implement health surveillance | Operators | Undetected progressive hearing loss | 3 | 4 | 12 | Audiometric testing arranged for employees regularly exposed ≥85 dB(A); baseline and periodic checks recorded; occupational health referral where required. | 1 | 4 | 4 |

## 11. Review & Monitoring

This GRA must be reviewed annually, following any change in equipment, introduction of new plant, significant noise monitoring results, or audit recommendation.