[COMPANY NAME]  –  EQUIPMENT INSPECTION

CHAINSAW INSPECTION

A monthly sheet for one chainsaw: a weekly inspection, plus a check after any repair, bar or chain change.

| At a glance |  |  |  |
| --- | --- | --- | --- |
| What it is for | Proving the saw is safe to use and has been inspected (PUWER 1998). | Who fills it in | A trained, competent chainsaw user or Team Leader. |
| How often | Weekly, after any repair or bar/chain change, and before first use. | Time needed | About 10 minutes, engine off and cool. |
| If it fails | Stop, tag it out, tell your Team Leader, record it below. | Where it goes | Equipment file for this saw. Keep for [at least 2 years]. |
| Reference | ARB-EQ-001 · Version 2.0 | Issued / review | [DD/MM/YYYY] / [DD/MM/YYYY] |

| Make and model |  | Asset / serial no. |  |
| --- | --- | --- | --- |
| Type |  | Month and year |  |
| Bar length / chain type |  | Assigned to |  |

Type of saw – tick all that apply

| ☐ Rear-handled | ☐ Top-handled (climbing saw) | ☐ Petrol | ☐ Battery |
| --- | --- | --- | --- |

How to mark each box:   ✓ = pass     ✗ = fail (do not use)     N/A = not fitted / not relevant

| Check | What good looks like | Wk 1 | Wk 2 | Wk 3 | Wk 4 | Wk 5 | Repair |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |
| Hours run (if known) |  |  |  |  |  |  |  |
| Safety features – never use the saw if any of these fail |  |  |  |  |  |  |  |
| Chain brake | Engages fully and stops the chain. Front hand guard moves freely and clicks on and off. |  |  |  |  |  |  |
| Throttle lockout | Throttle trigger will not move unless the lockout is pressed. |  |  |  |  |  |  |
| Stop switch | Clearly marked. Stops the engine or motor every time. |  |  |  |  |  |  |
| Chain catcher | Present, undamaged and secure. |  |  |  |  |  |  |
| Rear hand guard | Present, undamaged and secure. |  |  |  |  |  |  |
| Anti-vibration mounts | All present, not split, perished or loose. Handles do not feel "floppy". |  |  |  |  |  |  |
| Exhaust and spark arrestor | Secure, no cracks or holes; spark arrestor screen fitted and clear (petrol). |  |  |  |  |  |  |
| Cutting parts |  |  |  |  |  |  |  |
| Guide bar | Straight, rails even and free of burrs, groove clean, oil holes clear, nose sprocket turns freely. |  |  |  |  |  |  |
| Saw chain | Correct chain for the bar. Sharp, no cracked, loose or missing cutters, tie straps or links. |  |  |  |  |  |  |
| Chain tension | Correct: chain pulls round by hand (gloves on, engine off) and does not hang below the bar. |  |  |  |  |  |  |
| Drive sprocket and clutch | Sprocket not worn or grooved; clutch area clear of debris; clutch cover secure. |  |  |  |  |  |  |
| Bar nuts and cover | Present and tight. |  |  |  |  |  |  |
| Chain scabbard | Present, fits the bar and is not split. |  |  |  |  |  |  |
| Fuel, oil and power |  |  |  |  |  |  |  |
| Fuel tank, cap and lines (petrol) | No leaks, cap seals and locks, no fuel smell after refuelling. |  |  |  |  |  |  |
| Chain oil tank and oiler | No leaks; oil flicks off the chain tip when run over a light surface. |  |  |  |  |  |  |
| Air filter (petrol) | Clean and fitted correctly; cover secure. |  |  |  |  |  |  |
| Starter and recoil (petrol) | Cord not frayed; recoil returns fully. |  |  |  |  |  |  |
| Battery and terminals (battery) | Case not cracked or swollen; terminals clean; locks into the saw firmly. |  |  |  |  |  |  |
| Charger and connections (battery) | No damage, scorching or overheating. |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |
| Overall condition | Clean enough to inspect; handles, casing and screws secure; no cracks. |  |  |  |  |  |  |
| Labels and markings | Safety and noise labels legible; asset number visible. |  |  |  |  |  |  |
| Manufacturer features | Any other safety features in the handbook work as intended. |  |  |  |  |  |  |
| Checked by (initials) |  |  |  |  |  |  |  |

If anything fails (✗)

| 1 | STOP – do not use it | 2 | TAG IT OUT – label it "Do not use" and take it off the job | 3 | TELL your Team Leader straight away | 4 | RECORD it in the defect log below |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Every day before you start

The weekly inspection does not replace your quick daily checks. Before first use each day, check:

| ☐ Chain brake works | ☐ Throttle lockout works |
| --- | --- |
| ☐ Chain catcher is there | ☐ Chain is sharp and tensioned |
| ☐ Bar and chain are oiling | ☐ Stop switch works |
| ☐ No fuel or oil leaks | ☐ Your chainsaw PPE is in good condition |

Top-handled saws Top-handled saws are designed for use in the tree by trained climbers working from a rope and harness. Do not use them on the ground as a one-handed saw.

## Defect log

| Date | Defect found | Tagged out? | Action taken / by whom | Back in service (sign + date) |
| --- | --- | --- | --- | --- |

Monthly review. I have checked this sheet, and any defects above have been dealt with.

|  | Team Leader / Manager |
| --- | --- |
| Name |  |
| Signature |  |
| Date reviewed |  |

Guidance for the business owner – remove before issue Why this form matters. The Provision and Use of Work Equipment Regulations 1998 (PUWER) require work equipment to be maintained in a safe condition (reg 5), and inspected at suitable intervals where it can deteriorate and cause danger (reg 6). The results of inspections must be recorded and kept until at least the next inspection. Many businesses keep them for 2 years or more, which gives auditors a clear history. (Checked 23/09/2026.) Tailor it. Add your own asset numbers and delete rows that do not apply, for example the battery rows if you only use petrol saws. Follow the manufacturer’s handbook for service intervals, chain type and bar length. Guidance to follow. HSE tree work guidance, the Arboricultural Association (AA) Safety Guide 2 “Use of Tools in the Tree” (which replaced HSE leaflet AFAG 308 on top-handled saws), and the manufacturer’s instructions. Only trained, certificated operators should inspect and use chainsaws. Link it. Record chainsaw vibration levels and trigger times in your HAVS risk assessment and health surveillance (ARB-HR-011), and noise in your noise assessment (ARB-HR-012). Log the asset in your Register of Equipment (ARB-EQ-012). Arborella provides this template as documentation support. It does not replace the manufacturer’s instructions or a competent person’s judgement.

## Related documents

| Reference | Document |
| --- | --- |
| ARB-EQ-012 | Register of Equipment (noise and vibration) |
| ARB-EQ-000 | Equipment Defect Report and Log |
| ARB-EQ-013 | Do Not Use Tags |
| ARB-HR-009 | Technical (Site Safety) Induction Checklist |
| ARB-HR-011 | HAVS Health Surveillance Questionnaire |
| ARB-EQ-010 | Weekly Climbing Kit and PPE Inspection |
| [ref] | Chainsaw GRA and method statement |

## Document control

| Document | Chainsaw Inspection (ARB-EQ-001) |
| --- | --- |
| Owner | Company Director |
| Approved by | [Name, role] |
| Status | Controlled template – Live once approved |
| Review | At least every 12 months, and sooner if the law, guidance or the way you work changes |

| Ver. | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Initials] | First Edition Created |