[COMPANY NAME]  –  EQUIPMENT INSPECTION

STUMP GRINDER INSPECTION

A monthly sheet for one stump grinder: a weekly inspection, plus a check after any repair, service or cutter tooth change.

| At a glance |  |  |  |
| --- | --- | --- | --- |
| What it is for | Proving the grinder is safe to use and has been inspected (PUWER 1998). | Who fills it in | A trained, competent stump grinder operator or Team Leader. |
| How often | Weekly, after any repair, service or tooth change, and before first use. | Time needed | About 15 minutes. Most checks engine off, key out. |
| If it fails | Stop, tag it out, tell your Team Leader, record it below. | Where it goes | Equipment file for this grinder. Keep for [at least 2 years]. |
| Reference | ARB-EQ-003 · Version 2.0 | Issued / review | [DD/MM/YYYY] / [DD/MM/YYYY] |

| Make and model |  | Asset / serial no. |  |
| --- | --- | --- | --- |
| Registration / ID |  | Month and year |  |
| Trailer MAM (if towed) |  | Assigned to |  |

Type of grinder – tick all that apply

| ☐ Pedestrian | ☐ Tracked | ☐ Trailer-mounted | ☐ PTO-driven |
| --- | --- | --- | --- |

How to mark each box:   ✓ = pass     ✗ = fail (do not use)     N/A = not fitted / not relevant

| Check | What good looks like | Wk 1 | Wk 2 | Wk 3 | Wk 4 | Wk 5 | Repair |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |
| Hours run (if known) |  |  |  |  |  |  |  |
| Safety controls and guards – never use the grinder if any of these fail |  |  |  |  |  |  |  |
| Engine stop / emergency stop | Clearly marked, in reach from the operating position. Stops the engine every time. |  |  |  |  |  |  |
| Dead-man (operator presence) control | Let go and the cutter drive disengages. Cutter wheel stops in the time the handbook states. |  |  |  |  |  |  |
| Hold-to-run drive controls | Pedestrian and tracked: let go and the machine stops moving. Levers return to neutral. |  |  |  |  |  |  |
| Cutter wheel guard | Present, secure, not cracked or bent. Covers the wheel as designed. |  |  |  |  |  |  |
| Debris curtains / flaps | Present, intact and full length. Not torn, split or tied up out of the way. |  |  |  |  |  |  |
| Belt, shaft and fan guards | All present and secure on the machine and any power source. |  |  |  |  |  |  |
| PTO shaft and guard (PTO-driven) | Guard encloses the full length, is fitted correctly and works. PTO speed suits the machine. |  |  |  |  |  |  |
| Cutter wheel and teeth (engine off, key out, wheel stopped and secured) |  |  |  |  |  |  |  |
| Cutter teeth | All present. Tips not chipped, cracked or worn past the handbook limit. Matched set. |  |  |  |  |  |  |
| Tooth pockets | Not cracked, worn or bent. Each tooth sits tight in its pocket. |  |  |  |  |  |  |
| Tooth bolts and nuts | All present and torqued to the handbook. Locking nuts renewed when the handbook says. |  |  |  |  |  |  |
| Cutter wheel | No cracks, not bent. Turns freely without fouling. No wire or roots wrapped round. |  |  |  |  |  |  |
| Cutter shaft bearings | No play or grinding noise. |  |  |  |  |  |  |
| Drive belts | Correct tension, not cracked or glazed. |  |  |  |  |  |  |
| Engine and hydraulics |  |  |  |  |  |  |  |
| Fluid levels | Engine oil, coolant (where fitted) and hydraulic oil between the marks. |  |  |  |  |  |  |
| Fuel system | Secure. No leaks or damage. Cap seals. |  |  |  |  |  |  |
| Air filter and cooling | Filter serviceable. Radiator and screens clear of chips and dust. |  |  |  |  |  |  |
| Exhaust | Secure and intact. |  |  |  |  |  |  |
| Hydraulic hoses and fittings | No leaks, chafing or bulges, including the sweep and lift cylinders. |  |  |  |  |  |  |
| Sweep and lift controls | Operate smoothly and correctly. No creep when released. |  |  |  |  |  |  |
| Chassis, wheels, tracks and towing |  |  |  |  |  |  |  |
| Frame and chassis | No cracks, distortion or broken welds. |  |  |  |  |  |  |
| Tracks or tyres | Correct tension or pressure. No cuts or damage. Wheel nuts tight. |  |  |  |  |  |  |
| Wheel bearings (wheeled units) | No play. |  |  |  |  |  |  |
| Towing (trailer-mounted) | Complete the Trailer Inspection (ARB-EQ-008): hitch, safety chain or breakaway, lights. |  |  |  |  |  |  |
| Flying debris and site kit |  |  |  |  |  |  |  |
| Debris screens | Portable screens or boards present, not holed, and stand up firmly. |  |  |  |  |  |  |
| Barriers and signs | Enough to close off the exclusion zone. Signs readable. |  |  |  |  |  |  |
| Fire extinguisher | Present, in date, within easy reach. |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |
| Overall condition | Clean enough to inspect. No loose panels or missing fixings. |  |  |  |  |  |  |
| Warning devices and decals | Alarms or beacons work (where fitted). Safety decals readable. |  |  |  |  |  |  |
| Run test | No unusual noise, vibration or loss of power. |  |  |  |  |  |  |
| Checked by (initials) |  |  |  |  |  |  |  |

If anything fails (✗)

| 1 | STOP – do not use it | 2 | TAG IT OUT – label it "Do not use" and take it off the job | 3 | TELL your Team Leader straight away | 4 | RECORD it in the defect log below |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Every day before you start

The weekly inspection does not replace your daily checks. Before first use each day, check:

| ☐ Engine stop works | ☐ Dead-man control stops the cutter |
| --- | --- |
| ☐ Guards and debris curtains in place | ☐ Cutter teeth all there and tight |
| ☐ No fuel, oil or hydraulic leaks | ☐ Fire extinguisher to hand |
| ☐ PPE: helmet, eye protection or visor, hearing protection, boots, gloves, non-snag clothing | ☐ End of the job: clean off soil and plant debris before you leave site |

## On site: set up the flying-debris zone

| ☐ Underground services checked (plans, and a cable locator where needed) | ☐ Stump cleared of stones, metal and other debris |
| --- | --- |
| ☐ Exclusion zone of [x metres] set out with barriers and signs | ☐ Screens up where people, vehicles, windows or property are at risk |
| ☐ Chips aimed away from paths, roads and buildings | ☐ Nobody inside the zone except the operator |

Stay clear of the cutter Never approach the cutting wheel until it has stopped completely. Before working on the cutter: apply the engine stop, take the key out, and secure the cutter wheel so it cannot turn.

## Defect log

| Date | Defect found | Tagged out? | Action taken / by whom | Back in service (sign + date) |
| --- | --- | --- | --- | --- |

Monthly review. I have checked this sheet, and any defects above have been dealt with.

|  | Team Leader / Manager |
| --- | --- |
| Name |  |
| Signature |  |
| Date reviewed |  |

Guidance for the business owner – remove before issue Why this form matters. The Provision and Use of Work Equipment Regulations 1998 (PUWER) require work equipment to be suitable (reg 4), maintained in a safe condition (reg 5), and inspected at suitable intervals where it can deteriorate and cause danger (reg 6). Inspection results must be recorded and kept until at least the next inspection. Keeping them for 2 years or more gives auditors a clear history. (Checked 23/09/2026, legislation.gov.uk.) Set the exclusion zone. HSE guidance does not give a single distance. Fill in [x metres] from the manufacturer’s handbook and your site risk assessment, and make it larger near roads, footpaths, buildings and glass. Use screens wherever people or property could be hit. Tailor it. Add your asset number. Delete rows that do not apply, for example the PTO row on a self-powered machine. Use the handbook for tooth wear limits, bolt torques, the cutter stopping time and service intervals. Replace teeth as the handbook says so the wheel stays balanced. Guidance to follow. HSE leaflet AFAG606 (rev1) "Mobile stump grinders", still published by HSE (the Arboricultural Association has not yet replaced it with its own guide), and the manufacturer’s handbook. (Checked 23/09/2026, hse.gov.uk, trees.org.uk.) Only trained, competent operators should use or inspect stump grinders. Towing. For a trailer-mounted grinder, complete the Trailer Inspection (ARB-EQ-008) as well and check the driver is authorised to tow (ARB-HR-013). If the towing vehicle plus trailer could exceed 3,500 kg combined, operator licence or tachograph rules may apply. Check with DVSA or the Office of the Traffic Commissioner. Link it. Use the noise and vibration values from the handbook in your noise assessment (ARB-HR-012) and HAVS assessment (ARB-HR-011). Log the asset in your Register of Equipment (ARB-EQ-012). Cleaning down between sites supports your Biosecurity Policy ([ref]). Arborella provides this template as documentation support. It does not replace the manufacturer’s instructions or a competent person’s judgement.

## Related documents

| Reference | Document |
| --- | --- |
| ARB-EQ-012 | Register of Equipment (noise and vibration) |
| ARB-EQ-000 | Equipment Defect Report and Log |
| ARB-EQ-013 | Do Not Use Tags |
| ARB-EQ-008 | Trailer Inspection |
| ARB-HR-009 | Technical (Site Safety) Induction Checklist |
| ARB-HR-011 | HAVS Health Surveillance Questionnaire |
| ARB-HR-012 | Hearing (Noise) Health Surveillance Questionnaire |
| ARB-HR-013 | Driver Authorisation Form |
| [ref] | Stump grinding GRA and method statement |

## Document control

| Document | Stump Grinder Inspection (ARB-EQ-003) |
| --- | --- |
| Owner | Company Director |
| Approved by | [Name, role] |
| Status | Controlled template – Live once approved |
| Review | At least every 12 months, and sooner if the law, guidance or the way you work changes |

| Ver. | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Initials] | First edition created |