[COMPANY NAME]  –  EQUIPMENT INSPECTION

BRUSHCUTTER AND STRIMMER INSPECTION

A monthly sheet for one brushcutter or strimmer: a weekly inspection, plus a check after any repair, service or attachment change.

| At a glance |  |  |  |
| --- | --- | --- | --- |
| What it is for | Proving the machine is safe to use and has been inspected (PUWER 1998). | Who fills it in | A trained, competent user or Team Leader. |
| How often | Weekly, after any repair or attachment change, and before first use. | Time needed | About 10 minutes, engine off and cool, battery out. |
| If it fails | Stop, tag it out, tell your Team Leader, record it below. | Where it goes | Equipment file for this machine. Keep for [at least 2 years]. |
| Reference | ARB-EQ-004 · Version 2.0 | Issued / review | [DD/MM/YYYY] / [DD/MM/YYYY] |

| Make and model |  | Asset / serial no. |  |
| --- | --- | --- | --- |
| Attachments held |  | Month and year |  |
| Hours at start of month |  | Assigned to |  |

Type of machine – tick all that apply

| ☐ Brushcutter | ☐ Strimmer | ☐ Clearing saw | ☐ Split-shaft | ☐ Multi-tool |
| --- | --- | --- | --- | --- |
| ☐ Petrol | ☐ Battery | ☐ Mains | ☐ Loop handle | ☐ Bike handle |

How to mark each box:   ✓ = pass     ✗ = fail (do not use)     N/A = not fitted / not relevant

| Check | What good looks like | Wk 1 | Wk 2 | Wk 3 | Wk 4 | Wk 5 | Repair |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |
| Attachment fitted (line / blade / saw) |  |  |  |  |  |  |  |
| Safety features – never use the machine if any of these fail |  |  |  |  |  |  |  |
| Stop switch | Clearly marked. Stops the engine or motor every time. |  |  |  |  |  |  |
| Throttle trigger and lockout | Trigger will not move unless the lockout is pressed. Springs back to idle |  |  |  |  |  |  |
| Attachment stops at idle | Line head or blade stops turning at idle or when the trigger is released. |  |  |  |  |  |  |
| Correct guard for the attachment | The guard the handbook lists: line guard for a line head, blade guard for grass or brush blades, saw-blade guard for a clearing saw. |  |  |  |  |  |  |
| Guard condition | Not cracked, split or worn through. All fixings tight. |  |  |  |  |  |  |
| Harness quick-release | Opens fast with one hand and the machine drops clear. Test it weekly. |  |  |  |  |  |  |
| Barrier bar (loop handle) | Fitted and secure where the handbook requires it with a blade. |  |  |  |  |  |  |
| No chain or flail attachments | No attachment made of linked metal parts (for example chain flails) is fitted or kept with the machine. |  |  |  |  |  |  |
| Cutting attachment |  |  |  |  |  |  |  |
| Metal blade (if fitted) | Approved for this machine. Not bent. No cracks at tooth roots or centre hole. No missing teeth. Sharpened evenly. |  |  |  |  |  |  |
| Blade fixings | Correct thrust plate, washer and nut, the right way round. Nut tight. Blade runs true. |  |  |  |  |  |  |
| Line head (if fitted) | Not cracked or worn through. Eyelets not worn. Correct line size. Line-cutting knife on the guard. |  |  |  |  |  |  |
| Shaft, gear head, handles and harness |  |  |  |  |  |  |  |
| Shaft and drive tube | Straight, no cracks or dents. Split-shaft coupling locks with no play. |  |  |  |  |  |  |
| Gear head | Secure. Greased as the handbook says. No leaks, grinding or overheating. |  |  |  |  |  |  |
| Anti-vibration mounts | All present, not split, perished or loose. Handles do not feel "floppy". |  |  |  |  |  |  |
| Handles and controls | Secure and set as the handbook says. Grips not torn or loose. |  |  |  |  |  |  |
| Harness | Right type for the attachment (for example double-shoulder for blades). Stitching intact, buckles lock, hip pad fitted. |  |  |  |  |  |  |
| Hook and suspension point | Hook closes. Suspension eye secure. Machine hangs balanced. |  |  |  |  |  |  |
| Fuel, oil and power |  |  |  |  |  |  |  |
| Fuel tank, cap and lines (petrol) | No leaks or cracks. Cap seals. No fuel smell after refuelling. |  |  |  |  |  |  |
| Air filter, starter, exhaust (petrol) | Filter clean. Cord not frayed, recoil returns. Exhaust secure, not cracked. |  |  |  |  |  |  |
| Battery and charger (battery) | Not cracked or swollen. Terminals clean. Locks in firmly. Charger undamaged. |  |  |  |  |  |  |
| Cable and plug (electric) | No cuts, taped repairs or exposed wires. Plug not cracked. |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |
| Overall condition and labels | Clean enough to inspect. No cracks, screws secure. Safety and rotation labels legible. Asset number visible. |  |  |  |  |  |  |
| Test run | Starts and runs smoothly. No unusual vibration or noise. |  |  |  |  |  |  |
| Checked by (initials) |  |  |  |  |  |  |  |

If anything fails (✗)

| 1 | STOP – do not use it | 2 | TAG IT OUT – label it "Do not use" and take it off the job | 3 | TELL your Team Leader straight away | 4 | RECORD it in the defect log below |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Every day before you start

The weekly inspection does not replace your quick daily checks. Before first use each day, check:

| ☐ Stop switch works | ☐ Throttle lockout works |
| --- | --- |
| ☐ Attachment stops at idle | ☐ Right guard for the attachment, secure |
| ☐ Blade or line head undamaged | ☐ Harness quick-release opens |
| ☐ No fuel or oil leaks | ☐ Area walked for stones, wire and glass |
| ☐ Others at least [15 m – check handbook] away | ☐ PPE: visor, ear defenders, gloves, boots |

Changing the attachment Change the guard when you change the attachment. A line guard does not protect you from a metal blade. Only fit attachments the manufacturer approves for this machine. Never fit chain flails or other linked metal attachments: they can break up and throw metal parts.

## Defect log

| Date | Defect found | Tagged out? | Action taken / by whom | Back in service (sign + date) |
| --- | --- | --- | --- | --- |

Monthly review. I have checked this sheet, and any defects above have been dealt with.

|  | Team Leader / Manager |
| --- | --- |
| Name |  |
| Signature |  |
| Date reviewed |  |

Guidance for the business owner – remove before issue Why this form matters. The Provision and Use of Work Equipment Regulations 1998 (PUWER) require work equipment to be suitable (reg 4), maintained in a safe condition (reg 5), and inspected at suitable intervals where it can deteriorate and cause danger (reg 6). The results must be recorded and kept until at least the next inspection. Many businesses keep them for 2 years or more, which gives auditors a clear history. (Checked 23/09/2026, legislation.gov.uk.) Chain flails. HSE safety bulletin OPSTD 1-2013 warns that brushcutter attachments made of several linked metal parts, such as chain flails, can fail and throw metal parts, causing death or serious injury. HSE says to take machines fitted with them out of service, remove the attachment and fit the manufacturer’s approved accessory. (Checked 23/09/2026, hse.gov.uk.) Tailor it. Add your own asset numbers. Delete rows that do not apply, for example the petrol rows for battery machines. Use the manufacturer’s handbook to confirm which guard, harness and handle go with each attachment, the safe distance for other people, service intervals and gear head greasing. Guidance to follow. The manufacturer’s instructions and HSE tree work guidance on machinery. Only people trained and competent on brushcutters (and on clearing saws, if you use blades) should inspect and use them. Link it. Brushcutters and strimmers can give high vibration and noise. Put the handbook vibration and noise values into your HAVS risk assessment and health surveillance (ARB-HR-011) and your noise assessment (ARB-HR-012). Log the asset in your Register of Equipment (ARB-EQ-012). Arborella provides this template as documentation support. It does not replace the manufacturer’s instructions or a competent person’s judgement.

## Related documents

| Reference | Document |
| --- | --- |
| ARB-EQ-012 | Register of Equipment (noise and vibration) |
| ARB-EQ-000 | Equipment Defect Report and Log |
| ARB-EQ-013 | Do Not Use Tags |
| ARB-HR-009 | Technical (Site Safety) Induction Checklist |
| ARB-HR-011 | HAVS Health Surveillance Questionnaire |
| ARB-HR-012 | Hearing (Noise) Health Surveillance Questionnaire |
| [ref] | Brushcutter and strimmer GRA and method statement |

## Document control

| Document | Brushcutter and Strimmer Inspection (ARB-EQ-004) |
| --- | --- |
| Owner | Company Director |
| Approved by | [Name, role] |
| Status | Controlled template – Live once approved |
| Review | At least every 12 months, and sooner if the law, guidance or the way you work changes |

| Ver. | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Initials] | First edition created |