[COMPANY NAME]  –  EQUIPMENT INSPECTION

DAILY VEHICLE WALK-ROUND CHECK

One sheet per vehicle per week: the driver’s walk-round check each day before the vehicle is first used.

| At a glance |  |  |  |
| --- | --- | --- | --- |
| What it is for | Proving the vehicle is safe and roadworthy before it goes on the road. | Who fills it in | The driver, before first use each day. Only authorised drivers (ARB-HR-013). |
| How often | Every day the vehicle is used, and when the driver changes. | Time needed | About 10 minutes. |
| If it fails | Do not drive it. Tell your Team Leader, record who you told. | Where it goes | Vehicle file. Keep for [at least 15 months]. |
| Reference | ARB-EQ-009 · Version 2.0 | Issued / review | [DD/MM/YYYY] / [DD/MM/YYYY] |

| Registration |  | Week starting |  |
| --- | --- | --- | --- |
| Make and model |  | Main driver(s) |  |
| Plated GVW (kg) |  | Next service / MOT due |  |

Vehicle type – tick all that apply

| ☐ Van | ☐ Tipper | ☐ Pick-up | ☐ Chip-box | ☐ Other |
| --- | --- | --- | --- | --- |

How to mark each box:   ✓ = pass     ✗ = fail (do not use)     N/A = not fitted / not relevant

| Check | What good looks like | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |  |
| Mileage at start |  |  |  |  |  |  |  |  |
| Never drive if any of these fail |  |  |  |  |  |  |  |  |
| Brakes | Footwell clear. Footbrake and parking brake work, no excess travel. Vehicle holds on the handbrake. |  |  |  |  |  |  |  |
| Steering | Turns smoothly, no jamming or excess play. Power steering works. |  |  |  |  |  |  |  |
| Tyres and wheels | Tread at least 1.6 mm. Correctly inflated. No cuts, bulges or cord showing. Wheel nuts all there and secure. |  |  |  |  |  |  |  |
| Lights and indicators | All work, including brake lights. Lenses fitted, clean and the right colour. |  |  |  |  |  |  |  |
| Windscreen, wipers and washers | Screen not damaged or blocked. Wipers and washers clear it properly. |  |  |  |  |  |  |  |
| Mirrors and cameras | In place, secure, clean and not damaged. Cameras work. |  |  |  |  |  |  |  |
| Seat belts | No cuts or fraying. Lock when plugged in and retract properly. |  |  |  |  |  |  |  |
| Dashboard warning lights | All go out after starting. No brake, ABS, airbag, steering or engine warnings. |  |  |  |  |  |  |  |
| Horn | Works. |  |  |  |  |  |  |  |
| Cab, engine and fluids |  |  |  |  |  |  |  |  |
| Seats and cab | Seats secure. Tools, chainsaws and fuel cans stowed, not loose in the cab. |  |  |  |  |  |  |  |
| Fluid levels | Oil, coolant, brake fluid and washer fluid at the right level. AdBlue topped up (if fitted). |  |  |  |  |  |  |  |
| Leaks | No fuel, oil, coolant or brake fluid leaks under the vehicle. Fuel cap secure. |  |  |  |  |  |  |  |
| Battery and exhaust | Battery secure, not leaking. Exhaust secure, no leaks or excess smoke. |  |  |  |  |  |  |  |
| Safety equipment | First aid kit, fire extinguisher and spill kit present and in date. |  |  |  |  |  |  |  |
| Body, load, tipper and chip-box |  |  |  |  |  |  |  |  |
| Bodywork and doors | No new damage, sharp edges or loose panels. All doors shut and lock. |  |  |  |  |  |  |  |
| Number plates | Correct, secure, clean and legible. |  |  |  |  |  |  |  |
| Load restraint | Load cannot move. Straps, nets and lashing points in good condition. Bulkhead intact. |  |  |  |  |  |  |  |
| Weight | Not overloaded: within the plated gross vehicle weight. Wet chips and timber are heavy – if in doubt, weigh it. |  |  |  |  |  |  |  |
| Chip-box and tailgate | Tailgate, doors and side locks secure. Net or sheet fitted so chips cannot blow out. |  |  |  |  |  |  |  |
| Tipper body and hydraulics (if fitted) | Body fully down before driving. No hydraulic leaks; hoses not chafed. Tipper warning light off. |  |  |  |  |  |  |  |
| Tipper locks, pins and props (if fitted) | Locking pins in, safety prop present. Tailgate catches secure. |  |  |  |  |  |  |  |
| Towing (if a trailer is used today) |  |  |  |  |  |  |  |  |
| Tow bar and ball | Secure, not cracked or bent. Breakaway cable attachment point present. |  |  |  |  |  |  |  |
| Tow electrics | Socket not damaged. All trailer lights work when connected. Trailer checked on ARB-EQ-008. |  |  |  |  |  |  |  |
| Biosecurity |  |  |  |  |  |  |  |  |
| Clean between sites | Wheels, arches, cab mats and load area free of soil and plant debris, as your biosecurity procedure. |  |  |  |  |  |  |  |
| Driver initials |  |  |  |  |  |  |  |  |
| Defect reported to (initials) |  |  |  |  |  |  |  |  |

If anything fails (✗)

| 1 | STOP – do not drive it | 2 | TELL your Team Leader straight away | 3 | RECORD it below, with who you reported it to | 4 | WAIT until it is fixed and signed off |
| --- | --- | --- | --- | --- | --- | --- | --- |

Found a defect on the road? If a defect makes the vehicle dangerous, stop somewhere safe and call your Team Leader. Do not carry on until it is fixed. Report smaller defects at the end of the day, and record them below.

## Defect log

| Date | Defect found | Reported to (name) | Safe to drive? | Action taken / by whom | Fixed (sign + date) |
| --- | --- | --- | --- | --- | --- |

Monthly review. I have checked this sheet, and any defects above have been dealt with.

|  | Team Leader / Manager |
| --- | --- |
| Name |  |
| Signature |  |
| Date reviewed |  |

Guidance for the business owner – remove before issue Why this form matters. Vehicles must be kept roadworthy under the Road Vehicles (Construction and Use) Regulations 1986 and the Road Vehicles Lighting Regulations 1989. DVSA’s guidance "Carry out van daily walkaround checks" says the driver is responsible for making sure the van is safe to drive, dangerous defects must be fixed before continuing a journey, and using a van in a dangerous condition can lead to an unlimited fine and a prison sentence. A driver can get 3 penalty points for each tyre that is not safe and legal. The minimum tread for cars, light vans and light trailers is 1.6 mm across the central three-quarters of the tyre, all the way round. (Checked 23/09/2026: GOV.UK / DVSA.) Recording defects. DVSA requires drivers working under a goods vehicle operator licence to record the registration, date, defect, its assessment, their name and who they reported it to. This form does that for all your vehicles, which is good practice even without an operator licence. Heavier vehicles and towing. A tipper over 3,500 kg gross plated weight, or a pick-up or van towing a chipper trailer where the combined plated weights exceed 3,500 kg, may need a goods vehicle operator licence, and drivers’ hours and tachograph rules may apply. Exemptions exist, for example for trailers under 1,020 kg unladen carrying only your own goods. Check with DVSA or the Office of the Traffic Commissioner rather than assuming you are exempt. Operator-licensed vehicles over 3,500 kg need the DVSA HGV walkaround check and a planned maintenance system. DVSA’s "Guide to maintaining roadworthiness" says drivers’ defect reports must be kept for at least 15 months; we suggest the same for all your vehicles. (Checked 23/09/2026.) Tailor it. Delete rows that do not apply, for example tipper or AdBlue rows. Follow the vehicle handbook and your servicing schedule. Add the fuel isolation and safety checks for any electric or alternative-fuel vehicle. Link it. Each driver needs a current Driver Authorisation Form with licence checks (ARB-HR-013). Trailers go on ARB-EQ-008. Link to your Driving at Work and Biosecurity policies [ref], and log the vehicle in the Register of Equipment (ARB-EQ-012). HSE’s "Driving at work" guidance (INDG382) explains managing work-related road risk. Arborella provides this template as documentation support. It does not replace the manufacturer’s instructions or a competent person’s judgement.

## Related documents

| Reference | Document |
| --- | --- |
| ARB-EQ-012 | Register of Equipment (noise and vibration) |
| ARB-EQ-000 | Equipment Defect Report and Log |
| ARB-EQ-013 | Do Not Use Tags |
| ARB-EQ-008 | Trailer Inspection |
| ARB-HR-013 | Driver Authorisation Form |
| [ref] | Driving at Work and Biosecurity policies |

## Document control

| Document | Daily Vehicle Walk-round Check (ARB-EQ-009) |
| --- | --- |
| Owner | Company Director |
| Approved by | [Name, role] |
| Status | Controlled template – Live once approved |
| Review | At least every 12 months, and sooner if the law, guidance or the way you work changes |

| Ver. | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Initials] | First edition created |