[COMPANY NAME]  –  EQUIPMENT INSPECTION

LOADER INSPECTION

A monthly sheet for one compact or articulated loader: a weekly inspection, plus a check after any repair, service or modification.

| At a glance |  |  |  |
| --- | --- | --- | --- |
| What it is for | Proving the loader is safe and inspected (PUWER 1998; LOLER 1998 if it lifts loads). | Who fills it in | A trained, competent, authorised loader operator or Team Leader. |
| How often | Weekly, after any repair, service or modification, and before first use. | Time needed | About 15 minutes. Engine off, boom down or on its support. |
| If it fails | Stop, tag it out, tell your Team Leader, record it below. | Where it goes | Equipment file for this loader. Keep for [at least 2 years]. |
| Reference | ARB-EQ-011 · Version 2.0 | Issued / review | [DD/MM/YYYY] / [DD/MM/YYYY] |

| Make and model |  | Asset / serial no. |  |
| --- | --- | --- | --- |
| Registration (if road use) |  | Month and year |  |
| Rated operating capacity (kg) |  | LOLER exam due (if lifting) |  |

Type and use – tick all that apply

| ☐ Articulated loader | ☐ Skid-steer / tracked | ☐ Used for lifting loads | ☐ Used on the road |
| --- | --- | --- | --- |

How to mark each box:   ✓ = pass     ✗ = fail (do not use)     N/A = not fitted / not relevant

| Check | What good looks like | Wk 1 | Wk 2 | Wk 3 | Wk 4 | Wk 5 | Repair |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  |  |  |  |  |  |  |
| Hours run |  |  |  |  |  |  |  |
| Operator protection and controls – never use the loader if any of these fail |  |  |  |  |  |  |  |
| ROPS / FOPS / overhead guard | Fitted, no cracks, bends, drilled holes or welded repairs. Bolts tight. Folding frame raised and locked. |  |  |  |  |  |  |
| Seat belt | Latches and releases. Webbing not cut or frayed. Anchorages secure. |  |  |  |  |  |  |
| Seat and seat switch | Seat secure. Seat switch (where fitted) stops drive and hydraulics when you stand up. |  |  |  |  |  |  |
| Quick-hitch locking pins | Both pins go fully through the attachment plate and lock. Lever latched. Pins not bent or worn. |  |  |  |  |  |  |
| Brakes and parking brake | Machine stops promptly. Parking brake holds on a slope. |  |  |  |  |  |  |
| Steering and articulation joint | Smooth, no excessive play. Pivot pins and retainers in place. |  |  |  |  |  |  |
| Hydraulic controls | Return to neutral when released. No boom or bucket creep. |  |  |  |  |  |  |
| Emergency stop (where fitted) | Tested: stops the engine. Will not restart until reset. |  |  |  |  |  |  |
| Lifting, boom and attachments |  |  |  |  |  |  |  |
| Rated capacity chart | Decal present and readable on the machine. Operator knows the limit for this attachment. |  |  |  |  |  |  |
| Boom, arms and pins | No cracks or bends. Pins, bushes and retainers in place; no excessive play. |  |  |  |  |  |  |
| Attachment in use | Right one for the machine and job. Weight within rated capacity. No cracks; teeth, tines or grab intact. |  |  |  |  |  |  |
| Attachment hydraulics | Couplers clean, fully engaged, no leaks. Dust caps on spare couplers. |  |  |  |  |  |  |
| LOLER thorough examination | If used for lifting loads: report in date, next due date written above, defects dealt with. |  |  |  |  |  |  |
| Engine, hydraulics and fluids |  |  |  |  |  |  |  |
| Fluid levels | Engine oil, coolant, brake fluid and hydraulic oil between the marks. |  |  |  |  |  |  |
| Hydraulic hoses and fittings | No leaks, chafing, bulges or cracked outer cover. Securely clipped. |  |  |  |  |  |  |
| Fuel | Enough for the job. No leaks. Cap seals. |  |  |  |  |  |  |
| Air filter and radiator | Filter serviceable. Radiator clear of chips, leaves and dust. |  |  |  |  |  |  |
| Exhaust | Secure and intact. |  |  |  |  |  |  |
| Wheels, lights and warnings |  |  |  |  |  |  |  |
| Wheels and tyres (and spare) | Correct pressure and tread. No cuts or bulges. Wheel nuts tight. |  |  |  |  |  |  |
| Horn | Works. |  |  |  |  |  |  |
| Lights and beacon | Work and are clean. Road lights and number plate if used on the road. |  |  |  |  |  |  |
| Warning lights and gauges | Work. No warnings showing after start-up. |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |
| Bodywork and chassis | Secure, no cracks, distortion or leaks underneath. |  |  |  |  |  |  |
| Steps, handholds and cab area | Secure, clean and non-slip. No loose items in the footwell. |  |  |  |  |  |  |
| Decals | Safety decals and load chart readable. |  |  |  |  |  |  |
| Checked by (initials) |  |  |  |  |  |  |  |

If anything fails (✗)

| 1 | STOP – do not use it | 2 | TAG IT OUT – label it "Do not use" and take it off the job | 3 | TELL your Team Leader straight away | 4 | RECORD it in the defect log below |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Every day before you start

The weekly inspection does not replace your daily checks. Before first use each day, check:

| ☐ ROPS / FOPS in place and locked | ☐ Seat belt works, and you wear it |
| --- | --- |
| ☐ Walk round: no leaks or damage | ☐ Fluid levels and fuel |
| ☐ Brakes, steering and horn work | ☐ Lights and beacon work |
| ☐ Attachment fitted, pins locked (see below) | ☐ Load within the rated capacity for this attachment |

Every time you change an attachment Lock the quick-hitch, then look: both locking pins must be fully through the attachment plate. Then do the locking test the handbook describes before you lift. Connect and check the attachment hydraulics. Keep people clear, and carry loads low.

## Defect log

| Date | Defect found | Tagged out? | Action taken / by whom | Back in service (sign + date) |
| --- | --- | --- | --- | --- |

Monthly review. I have checked this sheet, and any defects above have been dealt with.

|  | Team Leader / Manager |
| --- | --- |
| Name |  |
| Signature |  |
| Date reviewed |  |

Guidance for the business owner – remove before issue Why this form matters. The Provision and Use of Work Equipment Regulations 1998 (PUWER) require work equipment to be suitable (reg 4), maintained (reg 5), and inspected at suitable intervals where it can deteriorate and cause danger (reg 6), with records kept until at least the next inspection. PUWER reg 26 covers protection where mobile work equipment could roll over, including a restraining system such as a seat belt where there is a risk of crushing. (Checked 23/09/2026, legislation.gov.uk.) If it lifts loads, LOLER applies. HSE treats loaders and fore-loaders used to lift loads as lifting equipment. They need a thorough examination by a competent person at least every 12 months (or in line with an examination scheme), and after exceptional circumstances such as damage. The examiner must tell you straight away about any defect that is an existing or imminent risk of serious injury, and send a copy of the report to the enforcing authority. Keep reports until the next report or for 2 years, whichever is later. This weekly check does not replace a thorough examination. (Checked 23/09/2026, HSE INDG422 and hse.gov.uk.) Ask your examiner which attachments the examination covers. Tailor it. Add your asset number and the rated operating capacity. Delete rows that do not apply, for example the seat switch or brake fluid. Follow the manufacturer’s handbook for the quick-hitch type, the load chart for each attachment, and service intervals. Add a row for any attachment you use a lot, for example a log grab or stump grinder head. Guidance to follow. The manufacturer’s handbook, HSE guidance on LOLER (INDG422 "Thorough examination of lifting equipment") and HSE guidance on safe use of mobile work equipment. Only trained, competent operators authorised by you should use or inspect the loader. If it is driven on the road, check registration, insurance and licence entitlement and complete the Driver Authorisation Form (ARB-HR-013). Link it. Use the noise values from the handbook in your noise assessment (ARB-HR-012), and the whole-body vibration values in your vibration risk assessment (the Control of Vibration at Work Regulations 2005 cover whole-body as well as hand-arm vibration). Log the asset and its LOLER due date in your Register of Equipment (ARB-EQ-012). Arborella provides this template as documentation support. It does not replace the manufacturer’s instructions or a competent person’s judgement.

## Related documents

| Reference | Document |
| --- | --- |
| ARB-EQ-012 | Register of Equipment (noise and vibration) |
| ARB-EQ-000 | Equipment Defect Report and Log |
| ARB-EQ-013 | Do Not Use Tags |
| ARB-EQ-008 | Trailer Inspection (for moving the loader on a trailer) |
| ARB-HR-009 | Technical (Site Safety) Induction Checklist |
| ARB-HR-012 | Hearing (Noise) Health Surveillance Questionnaire |
| ARB-HR-013 | Driver Authorisation Form |
| [ref] | Loader GRA and method statement |

## Document control

| Document | Compact and Articulated Loader Inspection (ARB-EQ-011) |
| --- | --- |
| Owner | Company Director |
| Approved by | [Name, role] |
| Status | Controlled template – Live once approved |
| Review | At least every 12 months, and sooner if the law, guidance or the way you work changes |

| Ver. | Date | Author | Summary of changes |
| --- | --- | --- | --- |
| 1.0 | [Date] | [Initials] | First edition created |