# COMPANY - COSHH ASSESSMENT

BIO CHAIN OIL (GENERIC)

| Assessment status | Document control |
| --- | --- |
| Generic - verify against exact product SDS | Reference: AOS-COSHH ¦ Version: 2.0 |
| Applies to | Assessment date: ____ / ____ / ______ |
| Arboriculture, tree surgery, workshop and site activities as relevant | Review date: ____ / ____ / ______ |

# Important: product-specific check

This assessment does not replace the manufacturer's current Safety Data Sheet (SDS) or product label. Record the exact product, attach the SDS and amend this assessment wherever the classification, limits, PPE, controls or emergency instructions differ.

# 1. Substance and task details

| Field | Details |
| --- | --- |
| Product / trade name | ____________________________________________ |
| Manufacturer / supplier | ____________________________________________ |
| SDS version / date | ____________________________________________ |
| Typical use | Filling chainsaw or pole-saw oil reservoirs and managing leaks during arboricultural work. |
| Activities assessed | Receipt, storage, transport, handling, application/dispensing, spill response and disposal, as applicable |
| Persons exposed | Operators, climbers, grounds staff, trainees/apprentices, contractors, visitors and the public if controls fail |
| Frequency / quantity | Record normal and maximum use: ______________________________ |

# 2. Typical CLP pictograms - verify against SDS

Many products of this type are not classified as hazardous under UK CLP. Do not add a pictogram unless it appears on the exact product label/SDS.

# 3. Main hazards and risk rating

| Hazard / route | Potential harm | Initial risk | Residual risk |
| --- | --- | --- | --- |
| Exposure / contact | Repeated skin contact may cause irritation or dermatitis depending on formulation. | High | Low |
| Task-related hazard | Splashes may irritate eyes. | Medium | Low |
| Environmental / secondary harm | Spilt oil creates a serious slip hazard on vehicles, platforms, steps, logs and ground. | Medium | Low |
| Fire / special hazard | Biodegradable does not mean harmless; releases can still pollute soil and water. | High | Low |

Residual ratings apply only when all controls below are in place. Stop work and inform the Team Leader if controls cannot be achieved.

# 4. Required controls

Fill on stable ground using a clean spout or funnel; do not fill while aloft.

Check caps, tanks and containers for leaks before transport and use.

Keep oil off handles, brakes, climbing equipment and PPE; clean contamination before use.

Use gloves for filling and cleaning; use eye protection where splashing is foreseeable.

Keep the current SDS available to the work team, brief relevant controls and record additional site-specific measures in the SSRA.

Use the smallest practicable quantity; keep containers closed, upright, labelled, undamaged and secured against unauthorised access.

Do not eat, drink or smoke while handling. Wash hands before breaks and after use. Remove contaminated clothing promptly.

Prevent entry into drains, watercourses and soil. Keep a suitable spill kit available and report all exposures, spills, leaks and near misses.

# 5. PPE and exposure control

| Control | Requirement |
| --- | --- |
| Gloves | Chemical-resistant gloves selected from the SDS for the product and task; inspect and replace if damaged or contaminated internally. |
| Eyes / face | Safety glasses or splash goggles/face protection where splashing, spray, drilling or pressurised release is foreseeable. |
| Clothing / footwear | Suitable work clothing and footwear; use protective coveralls or chemical-resistant clothing where required by the label/SDS. |
| Respiratory protection | Not a default control. Where required by the SDS/task assessment, select suitable RPE, provide face-fit testing for tight-fitting masks, training and maintenance. |
| Ventilation | Work outdoors or with adequate ventilation. Avoid breathing vapour, mist, spray, aerosol, dust or fumes. |

# 6. First aid and emergency action

| Exposure / incident | Immediate action |
| --- | --- |
| Skin | Remove contaminated clothing. Wash thoroughly with soap and water. Seek medical advice for persistent irritation, burns or other symptoms. |
| Eyes | Rinse cautiously with clean water for at least 10-15 minutes; remove contact lenses if easy. Follow SDS and obtain medical advice where required. |
| Inhalation | Move to fresh air, keep at rest and monitor. Seek medical advice if unwell; call 999 for breathing difficulty, collapse or severe symptoms. |
| Ingestion | Do not induce vomiting. Rinse mouth and obtain urgent medical advice. Show the label/SDS. |
| Spill | Stop work, isolate the area and remove ignition sources if safe. Stop the leak without risk, protect drains/water, absorb safely and dispose through an authorised route. |
| Fire / serious release | Raise alarm, evacuate, call 999 and keep upwind. Use an extinguisher only if trained, safe and there is a clear escape route. |

# 7. Storage, transport and disposal

Store in the original or approved compatible container in a secure, cool, well-ventilated location, segregated as required by the SDS.

Carry upright, capped and secured against movement or damage. Observe transport rules and do not leave hazardous products in a hot vehicle unnecessarily.

Never pour product into drains, onto land or into water. Dispose of surplus, packaging and contaminated absorbent according to the label/SDS and waste requirements.

Maintain storage inspections and stock control. Assess flammable storage/dispensing under DSEAR where relevant.

# 8. Training, checks and approval

| Required check |  | Record |  |
| --- | --- | --- | --- |
| Current product SDS/label attached and accessible |  | Yes / No |  |
| Workers trained/briefed and authorised for the task |  | Yes / No |  |
| Containers, equipment, spill kit and PPE checked |  | Yes / No |  |
| Site/environmental controls recorded in SSRA |  | Yes / No / N/A |  |
| Additional statutory competence/authorisation confirmed |  | Yes / No / N/A |  |
| Role | Name | Signature | Date |
| Assessor |  |  |  |
| Approved by |  |  |  |

Review at least annually and sooner after an SDS/product change, incident, ill-health report, significant task/storage change, or evidence that controls are ineffective.