# COMPANY - COSHH ASSESSMENT

BEE, WASP AND HORNET STINGS / INSECT BITES

| Assessment status | Document control |
| --- | --- |
| Hazard-specific assessment - confirm site exposure | Reference: AOS-BIO-08 ¦ Version: 1.0 |
| Applies to | Assessment date: ____ / ____ / ______ |
| Arboriculture, forestry, grounds maintenance and outdoor work | Review date: ____ / ____ / ______ |

# 1. Hazard identification

Common-wasp nest photograph: Wikimedia Commons.

| Hazard | Details |
| --- | --- |
| Common name | Bee, Wasp and Hornet Stings / Insect Bites |
| Organism / botanical name | Insect venom and allergens |
| Where encountered | Trees, cavities, hedges, ivy, sheds, ground nests and disturbed vegetation. |
| Persons potentially exposed | Employees, climbers, grounds staff, trainees/apprentices, contractors, visitors and the public if controls fail |

# 2. Warning symbols

Bite or sting

Naturally occurring biological or plant hazard; chemical CLP pictograms normally do not apply. The symbols used here are workplace warning and PPE symbols.

# 3. Exposure routes and health effects

| Exposure routes | Potential harm |
| --- | --- |
| Sting/bite; multiple stings; airborne or contact allergen | Pain, redness and swelling; Possible infection or life-threatening anaphylaxis |

| Initial risk | Residual risk after controls | Health surveillance |
| --- | --- | --- |
| High | Low | Review need with competent occupational-health advice where exposure is regular, sensitisation is possible or symptoms are reported. |

# 4. Required controls

Check client/site information and survey for this hazard before work. Record presence, extent, affected tasks and controls in the SSRA.

Survey for nests/flight activity; stop and isolate active nests

Do not disturb or treat nests unless trained and authorised

Workers with prescribed adrenaline must carry it and follow their individual plan

Use tools or mechanical methods to avoid direct contact where reasonably practicable. Maintain an exclusion zone where material could spread, project or expose others.

Do not eat, drink or smoke in the exposure area. Keep contaminated PPE/equipment away from vehicles, climbing equipment and welfare areas.

Provide soap-and-water handwashing and clean/decontaminate tools, boots and PPE without spreading contamination.

Stop work and inform the Team Leader if the hazard is unexpected, identification is uncertain or controls cannot be achieved.

# 5. Mandatory / task-selected PPE

| Coveralls / covered skin | Protective gloves | Eye protection |
| --- | --- | --- |

PPE selection must match the task and exposure level. RPE is only effective when correctly selected; tight-fitting RPE requires face-fit testing, a clean-shaven seal, training, inspection and replacement. PPE does not replace avoidance, mechanical control or exclusion.

# 6. Emergency and first aid action

Scrape a bee sting sideways; wash and apply a cold compress

Call 999 for breathing difficulty, facial/throat swelling, collapse or severe reaction; follow prescribed emergency plan

Report the exposure, injury, symptoms or near miss promptly and record the site, task, time and suspected agent.

Call 999 for breathing difficulty, collapse, severe allergic reaction, seizures, serious poisoning or other life-threatening symptoms.

# 7. Training, hygiene and review

| Required check | Record |
| --- | --- |
| Team trained to recognise the hazard and briefed on controls | Yes / No |
| Site-specific assessment and exclusion arrangements completed | Yes / No / N/A |
| PPE, washing, decontamination and first aid available | Yes / No / N/A |
| Specialist advice / occupational health required | Yes / No / N/A |
| Assessor / date / review date | ________________________________________ |

Review at least annually and sooner after an incident, symptom report, new guidance, change in work method, evidence of spread/contamination or concern that controls are ineffective.