# COMPANY - COSHH ASSESSMENT

OAK PROCESSIONARY MOTH / BROWN-TAIL MOTH

| Assessment status | Document control |
| --- | --- |
| Hazard-specific assessment - confirm site exposure | Reference: AOS-BIO-10 ¦ Version: 1.0 |
| Applies to | Assessment date: ____ / ____ / ______ |
| Arboriculture, forestry, grounds maintenance and outdoor work | Review date: ____ / ____ / ______ |

# 1. Hazard identification

OPM nest photograph: Forest Research / Forestry Commission.

| Hazard | Details |
| --- | --- |
| Common name | Oak Processionary Moth / Brown-Tail Moth |
| Organism / botanical name | Thaumetopoea processionea / Euproctis chrysorrhoea |
| Where encountered | Oak trees and OPM-contaminated bark, ground, tools and arisings; brown-tail webs on coastal scrub/hedges. |
| Persons potentially exposed | Employees, climbers, grounds staff, trainees/apprentices, contractors, visitors and the public if controls fail |

# 2. Warning symbols

Airborne irritant/allergen

Naturally occurring biological or plant hazard; chemical CLP pictograms normally do not apply. The symbols used here are workplace warning and PPE symbols.

# 3. Exposure routes and health effects

| Exposure routes | Potential harm |
| --- | --- |
| Airborne or direct contact with urticating hairs from caterpillars and nests | Itching rash, eye/throat irritation and breathing difficulty; Repeated exposure may cause sensitisation; hairs persist in nests/environment |

| Initial risk | Residual risk after controls | Health surveillance |
| --- | --- | --- |
| High | Low | Review need with competent occupational-health advice where exposure is regular, sensitisation is possible or symptoms are reported. |

# 4. Required controls

Check client/site information and survey for this hazard before work. Record presence, extent, affected tasks and controls in the SSRA.

Inspect from a safe distance and stop/isolate if suspected

Only competent specialists undertake direct nest work with a specific method/decontamination plan

Segregate contaminated tools, ropes and PPE; no dry brushing or compressed air

Use tools or mechanical methods to avoid direct contact where reasonably practicable. Maintain an exclusion zone where material could spread, project or expose others.

Do not eat, drink or smoke in the exposure area. Keep contaminated PPE/equipment away from vehicles, climbing equipment and welfare areas.

Provide soap-and-water handwashing and clean/decontaminate tools, boots and PPE without spreading contamination.

Stop work and inform the Team Leader if the hazard is unexpected, identification is uncertain or controls cannot be achieved.

# 5. Mandatory / task-selected PPE

| Coveralls / covered skin | Protective gloves | Eye protection | Suitable RPE | Protective footwear |
| --- | --- | --- | --- | --- |

PPE selection must match the task and exposure level. RPE is only effective when correctly selected; tight-fitting RPE requires face-fit testing, a clean-shaven seal, training, inspection and replacement. PPE does not replace avoidance, mechanical control or exclusion.

# 6. Emergency and first aid action

Withdraw, avoid rubbing, remove clothing carefully, wash/shower and flush eyes

Call 999 for breathing difficulty or severe allergic reaction; seek medical advice for persistent symptoms

Report the exposure, injury, symptoms or near miss promptly and record the site, task, time and suspected agent.

Call 999 for breathing difficulty, collapse, severe allergic reaction, seizures, serious poisoning or other life-threatening symptoms.

# 7. Training, hygiene and review

| Required check | Record |
| --- | --- |
| Team trained to recognise the hazard and briefed on controls | Yes / No |
| Site-specific assessment and exclusion arrangements completed | Yes / No / N/A |
| PPE, washing, decontamination and first aid available | Yes / No / N/A |
| Specialist advice / occupational health required | Yes / No / N/A |
| Assessor / date / review date | ________________________________________ |

Review at least annually and sooner after an incident, symptom report, new guidance, change in work method, evidence of spread/contamination or concern that controls are ineffective.