# COMPANY - COSHH ASSESSMENT

NETTLES

| Assessment status | Document control |
| --- | --- |
| Hazard-specific assessment - confirm site exposure | Reference: AOS-BIO-16 ¦ Version: 1.0 |
| Applies to | Assessment date: ____ / ____ / ______ |
| Arboriculture, forestry, grounds maintenance and outdoor work | Review date: ____ / ____ / ______ |

# 1. Hazard identification

Identification photographs retained from supplied SBust reference.

| Hazard | Details |
| --- | --- |
| Common name | Nettles |
| Organism / botanical name | Urtica dioica |
| Where encountered | Common in hedges, woodland edges, disturbed soil and nutrient-rich ground. |
| Persons potentially exposed | Employees, climbers, grounds staff, trainees/apprentices, contractors, visitors and the public if controls fail |

# 2. Warning symbols

Irritant / phototoxic sap

Naturally occurring biological or plant hazard; chemical CLP pictograms normally do not apply. The symbols used here are workplace warning and PPE symbols.

# 3. Exposure routes and health effects

| Exposure routes | Potential harm |
| --- | --- |
| Skin contact with stinging hairs; eye contact during cutting | Immediate burning, itching, redness and raised wheals |

| Initial risk | Residual risk after controls | Health surveillance |
| --- | --- | --- |
| High | Low | Review need with competent occupational-health advice where exposure is regular, sensitisation is possible or symptoms are reported. |

# 4. Required controls

Check client/site information and survey for this hazard before work. Record presence, extent, affected tasks and controls in the SSRA.

Identify dense growth; avoid bare-skin contact

Use gloves, long sleeves/trousers and eye protection for powered cutting

Keep others clear of projected fragments

Use tools or mechanical methods to avoid direct contact where reasonably practicable. Maintain an exclusion zone where material could spread, project or expose others.

Do not eat, drink or smoke in the exposure area. Keep contaminated PPE/equipment away from vehicles, climbing equipment and welfare areas.

Provide soap-and-water handwashing and clean/decontaminate tools, boots and PPE without spreading contamination.

Stop work and inform the Team Leader if the hazard is unexpected, identification is uncertain or controls cannot be achieved.

# 5. Mandatory / task-selected PPE

| Protective gloves | Eye protection | Coveralls / covered skin | Protective footwear |
| --- | --- | --- | --- |

PPE selection must match the task and exposure level. RPE is only effective when correctly selected; tight-fitting RPE requires face-fit testing, a clean-shaven seal, training, inspection and replacement. PPE does not replace avoidance, mechanical control or exclusion.

# 6. Emergency and first aid action

Wash gently and avoid rubbing/scratching; cool compress may help

Seek medical advice for eye contact, severe or persistent reaction

Report the exposure, injury, symptoms or near miss promptly and record the site, task, time and suspected agent.

Call 999 for breathing difficulty, collapse, severe allergic reaction, seizures, serious poisoning or other life-threatening symptoms.

# 7. Training, hygiene and review

| Required check | Record |
| --- | --- |
| Team trained to recognise the hazard and briefed on controls | Yes / No |
| Site-specific assessment and exclusion arrangements completed | Yes / No / N/A |
| PPE, washing, decontamination and first aid available | Yes / No / N/A |
| Specialist advice / occupational health required | Yes / No / N/A |
| Assessor / date / review date | ________________________________________ |

Review at least annually and sooner after an incident, symptom report, new guidance, change in work method, evidence of spread/contamination or concern that controls are ineffective.