# COMPANY - COSHH ASSESSMENT

SCREEN WASH (GENERIC)

| Assessment status | Document control |
| --- | --- |
| Generic - verify against exact product SDS | Reference: AOS-COSHH ¦ Version: 2.0 |
| Applies to | Assessment date: ____ / ____ / ______ |
| Arboriculture, tree surgery, workshop and site activities as relevant | Review date: ____ / ____ / ______ |

# Important: product-specific check

This assessment does not replace the manufacturer's current Safety Data Sheet (SDS) or product label. Record the exact product, attach the SDS and amend this assessment wherever the classification, limits, PPE, controls or emergency instructions differ.

# 1. Substance and task details

| Field | Details |
| --- | --- |
| Product / trade name | ____________________________________________ |
| Manufacturer / supplier | ____________________________________________ |
| SDS version / date | ____________________________________________ |
| Typical use | Diluting and filling vehicle or plant windscreen-washer reservoirs. |
| Activities assessed | Receipt, storage, transport, handling, application/dispensing, spill response and disposal, as applicable |
| Persons exposed | Operators, climbers, grounds staff, trainees/apprentices, contractors, visitors and the public if controls fail |
| Frequency / quantity | Record normal and maximum use: ______________________________ |

# 2. Typical CLP pictograms - verify against SDS

| Flammable | Harmful / irritant | Acute toxicity |
| --- | --- | --- |

These are typical possibilities for this product type. Use only the pictograms and signal word shown on the exact product label/SDS.

# 3. Main hazards and risk rating

| Hazard / route | Potential harm | Initial risk | Residual risk |
| --- | --- | --- | --- |
| Exposure / contact | Concentrated products may be flammable due to alcohol content. | High | Low |
| Task-related hazard | Methanol-containing products may be acutely toxic and may damage organs; formulation varies. | Medium | Low |
| Environmental / secondary harm | Splashes can irritate eyes and repeated skin contact may cause dryness. | Medium | Low |
| Fire / special hazard | Mist/vapour may cause symptoms in poorly ventilated areas. | High | Low |

Residual ratings apply only when all controls below are in place. Stop work and inform the Team Leader if controls cannot be achieved.

# 4. Required controls

Check the SDS for alcohol type and concentration; use a less hazardous methanol-free product where reasonably practicable.

Dilute only as directed and never transfer into drink containers.

Fill outdoors or with good ventilation, away from ignition sources, using a funnel or controlled spout.

Wear gloves for repeated handling and eye protection where splashing is foreseeable.

Keep the current SDS available to the work team, brief relevant controls and record additional site-specific measures in the SSRA.

Use the smallest practicable quantity; keep containers closed, upright, labelled, undamaged and secured against unauthorised access.

Do not eat, drink or smoke while handling. Wash hands before breaks and after use. Remove contaminated clothing promptly.

Prevent entry into drains, watercourses and soil. Keep a suitable spill kit available and report all exposures, spills, leaks and near misses.

# 5. PPE and exposure control

| Control | Requirement |
| --- | --- |
| Gloves | Chemical-resistant gloves selected from the SDS for the product and task; inspect and replace if damaged or contaminated internally. |
| Eyes / face | Safety glasses or splash goggles/face protection where splashing, spray, drilling or pressurised release is foreseeable. |
| Clothing / footwear | Suitable work clothing and footwear; use protective coveralls or chemical-resistant clothing where required by the label/SDS. |
| Respiratory protection | Not a default control. Where required by the SDS/task assessment, select suitable RPE, provide face-fit testing for tight-fitting masks, training and maintenance. |
| Ventilation | Work outdoors or with adequate ventilation. Avoid breathing vapour, mist, spray, aerosol, dust or fumes. |

# 6. First aid and emergency action

| Exposure / incident | Immediate action |
| --- | --- |
| Skin | Remove contaminated clothing. Wash thoroughly with soap and water. Seek medical advice for persistent irritation, burns or other symptoms. |
| Eyes | Rinse cautiously with clean water for at least 10-15 minutes; remove contact lenses if easy. Follow SDS and obtain medical advice where required. |
| Inhalation | Move to fresh air, keep at rest and monitor. Seek medical advice if unwell; call 999 for breathing difficulty, collapse or severe symptoms. |
| Ingestion | Do not induce vomiting. Rinse mouth and obtain urgent medical advice. Show the label/SDS. |
| Spill | Stop work, isolate the area and remove ignition sources if safe. Stop the leak without risk, protect drains/water, absorb safely and dispose through an authorised route. |
| Fire / serious release | Raise alarm, evacuate, call 999 and keep upwind. Use an extinguisher only if trained, safe and there is a clear escape route. |

# 7. Storage, transport and disposal

Store in the original or approved compatible container in a secure, cool, well-ventilated location, segregated as required by the SDS.

Carry upright, capped and secured against movement or damage. Observe transport rules and do not leave hazardous products in a hot vehicle unnecessarily.

Never pour product into drains, onto land or into water. Dispose of surplus, packaging and contaminated absorbent according to the label/SDS and waste requirements.

Maintain storage inspections and stock control. Assess flammable storage/dispensing under DSEAR where relevant.

# 8. Training, checks and approval

| Required check |  | Record |  |
| --- | --- | --- | --- |
| Current product SDS/label attached and accessible |  | Yes / No |  |
| Workers trained/briefed and authorised for the task |  | Yes / No |  |
| Containers, equipment, spill kit and PPE checked |  | Yes / No |  |
| Site/environmental controls recorded in SSRA |  | Yes / No / N/A |  |
| Additional statutory competence/authorisation confirmed |  | Yes / No / N/A |  |
| Role | Name | Signature | Date |
| Assessor |  |  |  |
| Approved by |  |  |  |

Review at least annually and sooner after an SDS/product change, incident, ill-health report, significant task/storage change, or evidence that controls are ineffective.