COMPANY

GENERIC RISK ASSESSMENT (GRA) – ENVIRONMENTAL PROTECTION & BIODIVERSITY

Activity / Task: Arboricultural and vegetation management operations with environmental, ecological and pollution risk considerations

Document Ref: ARB-GRA-28

Version: ____    Date Issued: ____    Review Date: ____

Assessed by: ______________________________

## 1. Scope of Risk Assessment

This Generic Risk Assessment (GRA) addresses environmental protection, pollution prevention and biodiversity considerations associated with arboricultural and vegetation management works. It applies to all operational activities including works near watercourses, protected sites, highways, domestic premises and commercial developments. This document must be read alongside Site-Specific Risk Assessments (SSRAs), task-specific GRAs and any client environmental specifications.

## 2. Responsibilities

Managers must ensure environmental risks are identified during planning, appropriate mitigation measures are implemented and compliance with environmental legislation is maintained.

Team Leaders must brief environmental controls during site induction, monitor compliance, and retain stop-work authority if environmental harm is likely.

Operatives must follow pollution prevention measures, report spills immediately and avoid unnecessary environmental disturbance.

## 3. Persons at Risk

☒ Environment    ☒ Wildlife    ☒ Watercourses    ☒ Public    ☒ Employees

## 4. PPE Requirements

PPE determined via SSRA and task-specific GRA. Spill response gloves and eye protection to be available when handling fuels or chemicals.

## 5. Emergency Arrangements

Spill kits must be available on site where fuels or oils are present. In the event of significant pollution, works must cease and the Environment Agency incident hotline (0800 80 70 60) contacted where applicable. Contaminated materials must be contained and disposed of via licensed waste carriers.

## 6. Risk Matrix & Scoring

Risk is assessed using Likelihood (L) and Severity (S). Risk Rating is calculated as L × S. Initial risk represents risk before controls; residual risk represents risk remaining once controls are applied.

| Severity ↓ / Likelihood → | 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Very Likely |
| --- | --- | --- | --- | --- | --- |
| 1 Minor | 1 | 2 | 3 | 4 | 5 |
| 2 First Aid | 2 | 4 | 6 | 8 | 10 |
| 3 Medical | 3 | 6 | 9 | 12 | 15 |
| 4 Major | 4 | 8 | 12 | 16 | 20 |
| 5 Fatal | 5 | 10 | 15 | 20 | 25 |

Risk bands: 1–4 Low | 5–9 Medium | 10–16 High | 17–25 Intolerable

## 7. Relevant UK Legislation & Guidance

Health and Safety at Work etc. Act 1974

Environmental Protection Act 1990

Wildlife and Countryside Act 1981

Countryside and Rights of Way Act 2000

Natural Environment and Rural Communities Act 2006

Environment Act 2021 (Biodiversity Net Gain provisions)

Control of Substances Hazardous to Health Regulations 2002 (COSHH)

Waste (England and Wales) Regulations 2011

Hedgerows Regulations 1997

Water Resources Act 1991

## 8. Hazards & Control Measures

| Hazard | Persons at Risk | How Harm May Occur | Initial L | Initial S | Initial L×S | Control Measures (Detailed) | Residual L×S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel or oil spill during refuelling | Environment, watercourses | Soil and water contamination | 3 | 4 | 12 | Refuel away from drains and watercourses; drip trays used; spill kits available; plant checked for leaks; contaminated materials disposed via licensed carrier; major spills reported to EA. | 4 |
| Works near watercourses | Water environment, wildlife | Siltation or contamination | 3 | 4 | 12 | Maintain buffer zones; no refuelling within 10m of water; spill kits available; follow ARB-GRA-22; cease work if pollution risk identified. | 4 |
| Disturbance of nesting birds or protected species | Wildlife | Legal breach under Wildlife & Countryside Act | 3 | 4 | 12 | Pre-works ecological checks; no disturbance of active nests; works rescheduled if required; licences obtained where necessary; staff briefed on protected species awareness. | 4 |
| Damage to trees protected by TPO or within Conservation Areas | Environment, company | Legal breach and enforcement action | 3 | 4 | 12 | Check TPO and conservation status prior to works; written consent obtained where required; works undertaken in accordance with approved specification. | 4 |
| Invasive species spread (e.g. Japanese Knotweed) | Environment | Ecological harm and legal breach | 3 | 4 | 12 | Identify invasive species prior to works; avoid disturbance unless authorised; follow Environment Agency guidance; dispose of contaminated material lawfully; clean tools before leaving site. | 4 |
| Improper waste disposal | Environment | Pollution and prosecution | 3 | 4 | 12 | Waste segregated; green waste processed appropriately; transfer notes retained; only licensed waste carriers used; no fly-tipping tolerated. | 4 |
| Soil compaction and root damage from plant | Trees, soil ecology | Long-term environmental damage | 3 | 3 | 9 | Use ground protection mats where required; avoid unnecessary trafficking; designate plant routes; avoid root protection areas unless agreed. | 4 |
| Failure to consider Biodiversity Net Gain requirements | Environment, client | Contractual non-compliance | 2 | 4 | 8 | Review planning conditions; liaise with client where BNG applies; document mitigation measures; retain ecological communication records. | 3 |

## 9. Review & Monitoring

This GRA must be reviewed annually or following any environmental incident, legislative update or change in operational practice.